Zebra ZXP Series 1
Card Printer

Specifications are provided for reference and are based
on printer tests using Genuine Zebra® supplies. Results may
vary in actual application settings or when using other-than-recommended Zebra supplies. Zebra recommends always qualifying any application with thorough testing.

Printing Specifications

- Dye-sublimation thermal transfer direct to card

- Full color or monochrome printing

- Single-sided printing

- 545 cph monochrome

- 130 cph YMCKO

- Edge-to-edge printing on standard CR-80 media

Standard features

- Eco-Friendly Load-N-Go™ drop-in ribbon ZXP Series 1™ cartridges

- ZRaster™ host-based image processing

- Auto calibration of ribbon

- USB 2.0 connectivity

- Microsoft ® Certified Windows drivers

- Energy Star Certified

- Print Touch™ NFC tag for online printer documentation access

- 100 card covered feeder (30 mil)

- 45 card capacity output hopper (30 mil)

- 16-character LCD operator control display

- 300 dpi (11.8 dots / mm) print resolution

- Two-year warranty on printer

- Two-year (unlimited passes) original printhead

- One-year hot swap (U.S. only)

Optional Features

- CardSense™ single-card feed mode (field upgradable)

- ZebraCare™ extended product warranty programs

Encoding Options and Specifications

Magnetic stripe encoder – ISO 7811 (new and re-encoded; tracks 1, 2, and 3; high and low coercivity; stripe down; 30 mil card only; factory install only)

Printing ThroughPUT

Printing throughput (cards per hour) based on click to drop printing, average print coverage, with USB connectivity. Times may vary depending on the computer and other configurations.

| Attribute | Ribbon (front) | Printing | Printing and Mag Encoding |
| --- | --- | --- | --- |
| Click to Drop (seconds) | YMCKO | 30 | 32 |
|  | Mono K | 9 | 12 |
| Throughput (cards / hour) | YMCKO | 130 | 125 |
|  | Mono K | 545 | 405 |

Card Compatibility

NOTE: For optimum print quality and printer performance,
use of Zebra genuine supplies is recommended.

- Card Thickness: 10-40 mil

- Card Size: CR-80, ISO 7810 format, Type ID-1, 2.12" (54 mm)
x 3.38" (86 mm)

- Card Material: PVC and PVC composite

- Spot Color or monochrome print only on < 20 mil card thickness

Communications Interfaces

- USB V2.0 (cable included) - standard

- Built-in 10/100 Ethernet (field upgradable) - optional

Physical SPECIFICATIONS 
(standard mode)

| Attribute | ZXP Series 1 |
| --- | --- |
| Height | 9.3" (236 mm) |
| Width | 7.9" (200 mm) |
| Depth: | 13.0" (330 mm) |
| Weight: | 11.6 lbs. (5.3 kg) |

Electrical Specifications

- Auto-switching single-phase AC power

- Operating range: 90-132VAC and 190-264VAC RMS

- Frequency range: 47-63 Hz

- FCC Class A

Supplies Specifications

NOTE: The ZXP Series 1 printer is designed to work only with Zebra True Colours® ix Series™, ZXP Series 1 ribbons.

- Zebra ix Series intelligent technology automatically detects
and authenticates ribbon

- Integrated cleaning roller included with each ribbon

- Media Starter Kit – 1 YMCKO ribbon, 100 30-mil PVC cards

- Cleaning Kit: 4 print engine cleaning cards and 4 feeder cleaning cards, 1000 images / card

True Colours ix series ZXP series 1 ribbon options

| Ribbon Description | Eco Cartridge Image Count |
| --- | --- |
| YMCKO | 100 |
| Black Monochrome | 1000 |
| White Monochrome | 500 |

Preventative Maintenance

Zebra recommends cleaning the printer on a regular basis using standard Zebra cleaning supplies and parts. Consult the User’s Guide for further details.

environmental Specifications

- Operating temperature: 60º F to 86º F (15º C to 30º C)

- Storage temperature: 23º F to 131º F (-5º C to 55º C)

- Operating humidity: 20% to 65% non-condensing

- Storage humidity: 20% to 70% non-condensing

- Media storage:

  - Temperature: 41º F to 77º F (5º C to 30º C)

  - Humidity: 35% to 65% non-condensing

  - Do not store in direct sunlight

Drivers supported

Microsoft Windows Certified Printer Drivers1

Windows XP, Windows Vista ®, Windows Server ® 2003, Windows Server 2008, Windows 7 (32-bit and 64-bit compatible), Windows 8 (32-bit and 64-bit compatible)

Please see zebra.com for up-to-date listing of supported systems.

1Requires Firmware Release 2.00 or greater